LED Gas Station Light

Specifications

| Product Series | LED Gas Station Light |
| --- | --- |
| Model No. | PTGA01A0/PTGA01A5 |
| Version | V3 |
| Date | 2023/2/20 |

Advantages

- Unique flat form design

- Die-casting integrated AL casing make faster heat dissipation

- Optical glass over 90% light transmitting

- LED 3030 chip with low lumen decay

- IP65 driver with short circuit/over-flow/over-temperature/over-voltage protection

- 120lm/W, over 90% energy saving

- Lifespan 50,000 hours, 5 years’ warranty

- Easy installation and low maintenance

Product Size

100W

PTHK01B0

150W

Unit: mm

Specifications

| Model No. | PTGA01A0 | PTGA01A5 |
| --- | --- | --- |
| Input voltage | AC100-277V | AC100-277V |
| Power | 100W | 150W |
| Dimming | Not | Not |
| PF | >0.95 | >0.95 |
| LED Type | SMD3030 | SMD3030 |
| Beam angle | 120° | 120° |
| Light efficiency | 120 lm/W | 120 lm/W |
| luminous | 12000 lm | 18000 lm |
| Color Temp | 3000±200K / 4000±250K / 5700±300K / 6500±500 K | 3000±200K / 4000±250K / 5700±300K / 6500±500 K |
| CRI | 70/80Ra | 70/80Ra |
| IP | 65 | 65 |
| Working Temp | -20°C~45°C | -20°C~45°C |
| Storage Temp | -40°C~60°C | -40°C~60°C |
| Life span | 50000h@25°C | 50000h@25°C |

Installation

CUT-OUT(100W:262mm 150W:330mm)

Package

| Model No. | PT-GS100W | PT-GS150W |
| --- | --- | --- |
| QTY/Carton | 1 pcs | 1 pcs |
| Carton Size | 350*350*210mm | 440*440*210mm |
| N.W | 4.1KG | 5.1KG |
| G.W | 5.4 KG | 6.7 KG |
| 20'' GP Container | 1010PCS | 640PCS |
| 40'' HQ Container | 2565PCS | 1620PCS |