SPORTS&LED HIGH MAST LIGHT

A Series

Specifications

| Product Series | Sports & High Mast Light-A Series |
| --- | --- |
| Model No. | PTMA04B5/ PTMA04E0/ PTMA04G5/ PTMA04J0/PTMA04L5 |
| Version | V1 |
| Date | 2023/02/23 |

Advantages

- ADC12 integrated die casting heat sink: excellent thermal management &compact structure

- Module design：reduces wind area resistance & Increases heat dissipation performance & dIY

module quantity

- Die-casting aluminum driver cover guarantee the corrosion resistance, easy for maintenance and driver protection

- Handle bar on both side: easy for installation & carry

- Racked angle: allows for precise Installation, Up to 270°

- IK10 high strength polycarbonate lens , 30°/60°/90°lens available and changeable

- Visor available, effectively control the spill light, upward light and avoid the lighting pollution.

- Aiming device available, allows for precise aiming and site installation

- Double surge protection: driver & extra 10KV SPD

- IP65 driver with short circuit/over-flow/over-temperature/over-voltage protection

- LED 3030 chip with high lumen efficiency and low lumen decay

- Lifespan 50,000 hours, 5 years’ warranty

- Easy installation and low maintenance

Product Size

PTMA04B5 250W                       PTMA04E0 500W

PTMA04G5 750W                     PTMA04J0 1000W

PTMA04L5

Unit: mm

Specifications

| Model No. | PTMA04B5 |  | PTMA04E0 |  | PTMA04G5 |  | PTMA04J0 | PTMA04L5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Input Voltage | AC100-277V |  |  |  |  |  |  |  |  |
| Power | 250W | 500W |  | 750W |  | 1000W |  |  | 1250W |
| Dimming | 1-10 DIM |  |  |  |  |  |  |  |  |
| PF | >0.95 |  |  |  |  |  |  |  |  |
| Power Supply | SOSEN(Flicker Free) |  |  |  |  |  |  |  |  |
| LED Type | 3030 |  |  |  |  |  |  |  |  |
| Beam Angle | 30°/60°/90° |  |  |  |  |  |  |  |  |
| Light Efficiency | 150 lm/W |  |  |  |  |  |  |  |  |
| Luminous | 37500 lm |  | 75000 lm |  | 112500 lm |  | 150000 lm | 187500 lm |  |
| Color Temp | 3000±200K /4000±200K /5000±250K/6000±250K |  |  |  |  |  |  |  |  |
| CRI | >70Ra（80Ra） |  |  |  |  |  |  |  |  |
| IK | IK10 |  |  |  |  |  |  |  |  |
| IP | 66 |  |  |  |  |  |  |  |  |
| SPD | Driver 10KV + Extra 10KV |  |  |  |  |  |  |  |  |
| Working Temp | -20°C~45°C |  |  |  |  |  |  |  |  |
| Storage Temp | -40°C~60°C |  |  |  |  |  |  |  |  |
| Life Span | 50000h@25°C |  |  |  |  |  |  |  |  |

Light Distribution Curve

| 30° | 60° | 90° |
| --- | --- | --- |
| AAI Figure |  |  |

Optional Accessories

Driver box              Surge Protective Device             Visor             Aiming Device(Laser Pen)

Installation