SPORTS & High MAST LIGHT

C Series

Specifications

| Product Series | Sports & High Mast Light-C Series |
| --- | --- |
| Model No. | PTMA01D0/PTMA01D0/PTMA01H0/PTMA01J0 |
| Version | V3 |
| Date | 2020/8/6 |

Advantages

- ADC12 integrated die casting heat sink: excellent thermal management &compact structure

- Module design：reduces wind area resistance & Increases heat dissipation performance

- Die-casting aluminum driver cover guarantee the corrosion resistance, easy for maintenance and driver protection

- Racked Angle: allows for precise installation, up to 270°

- IK10 high strength polycarbonate lens , 10°/25°/45°/60°/90° lens available and changeable

- Visor available, effectively control the spill light, upward light and avoid the lighting pollution.

- Aiming device available, allows for precise aiming and site installation

- Double surge protection: driver &extra 10KV SPD

- IP65 driver with short circuit/over-flow/over-temperature/over-voltage protection

- LED 3030 chip with high lumen efficiency and low lumen decay

- Lifespan 50,000 hours, 5 years’ warranty

- Easy installation and low maintenance

Product Size

PTMA01D0/PTMA01E0

PTMA01H0/PTMA01J0

Unit: mm

Specifications

| Model No. | PTMA01D0 | PTMA01E0 | PTMA01H0 | PTMA01J0 |
| --- | --- | --- | --- | --- |
| Input Voltage | AC100-277V | AC100-277V | AC100-277V | AC100-277V |
| Power | 400W | 500W | 800W | 1000W |
| Dimming | Not | Not | Not | Not |
| PF | >0.95 | >0.95 | >0.95 | >0.95 |
| Power Supply | SOSEN(Flicker Free) |  |  |  |
| LED Type | 770pcs*3030 | 770pcs*3030 | 1540pcs*3030 | 1540pcs*3030 |
| Beam Angle | 10°25°45°60°90° | 10°25°45°60°90° | 10°25°45°60°90° | 10°25°45°60°90° |
| Light Efficiency | 140 lm/W | 140 lm/W | 140 lm/W | 140 lm/W |
| Luminous | 56000 lm | 70000 lm | 112000 lm | 140000 lm |
| Color Temp | 3000±200K / 4000±250K / 5000±250K / 6500±500 K | 3000±200K / 4000±250K / 5000±250K / 6500±500 K | 3000±200K / 4000±250K / 5000±250K / 6500±500 K | 3000±200K / 4000±250K / 5000±250K / 6500±500 K |
| CRI | >70Ra (80ra) | >70Ra (80ra) | >70Ra (80ra) | >70Ra (80ra) |
| IP | 66 | 66 | 66 | 66 |
| Working Temp | -20°C~45°C | -20°C~45°C | -20°C~45°C | -20°C~45°C |
| Storage Temp | -40°C~60°C | -40°C~60°C | -40°C~60°C | -40°C~60°C |
| Life Span | 50000h@25°C | 50000h@25°C | 50000h@25°C | 50000h@25°C |

Light Distribution Curve

| PTMA01E0 | PTMA01J0 |
| --- | --- |

Installation