LED Linear Light

Specifications

| Product Series | LED Linear light B |
| --- | --- |
| Model No. | PTHL02A0/PTHL02A5/PTHL02B0 |
| Version | V2 |
| Date | 2022/12/08 |

Advantages

Advantages

- Pure aluminum 6063 heat sink with good heat dissipation, dusting process makes it UV resistant;

- New structure design, avoiding the cleaning pressure from accumulating water and dust and maintenance;

- Various beam angles are available, easy change lens according to application requirement;

- Various wattages can be made up, from 100w up to 200w. If need other wattages, please consult sales.

- LM80 certified 2835 light source;

- 150 lm/W, over 90% energy saving;

- Easy installation and low maintenance, suspension and surface installation are available.

Application

Application

To replace the conventional rectangle high-bay light anywhere at like: high roof warehouse, Promenade, stadium, aisle, shelf warehouse etc.

Product Size

Product Size

| PTHL02A0 |
| --- |
| PTHL02A5 | PTHL01A5 |
| PTHL02B0 |

Accessories

Accessories

Specifications

Specifications

| Model No. | PTHL02A0 | PTHL02A5 | PTHL02B0 |
| --- | --- | --- | --- |
| Input Voltage | AC100-277V |  |  |
| Power | 100W | 150W | 200W |
| Dimming | NO / 1-10V Dimmable |  |  |
| PF | >0.95 |  |  |
| Power Supply | SOSEN（Flicker Free） |  |  |
| LED Type | 2835 (LM80) |  |  |
| Beam Angle | 60° / 90° / 80*150° / 90*120° |  |  |
| Light Efficiency | 150LM/W |  |  |
| Luminous | 15000lm | 22500lm | 30000lm |
| Color Temp | 3000±200K / 4000±250K / 5700±300K / 6500±500K |  |  |
| CRI | >70 (Ra80 optional) |  |  |
| IP | IP65 |  |  |
| IK | IK08 |  |  |
| Working Temp | -20℃~45℃ |  |  |
| Storage Temp | -40℃~60℃ |  |  |
| Surge Protection | 6kV line-line, 6kV line-earth |  |  |
| Life Span | 50000h@25℃ |  |  |

Installation

Installation

Package

Package

| Model No. | PTHL02A0 | PTHL02A5 | PTHL02B0 |
| --- | --- | --- | --- |
| QTY/Carton | 1 pcs | 1pcs | 1 pcs |
| Carton Size | 65.5*20.5*14.5cm | 95.5*20.5*14.5cm | 125.5*20.5*14.5cm |
| N.W | 2.2 kg | 3.1 kg | 3.8 kg |
| G.W | 2.6 kg | 3.6 kg | 4.5 kg |
| 20''GP Container | 1330pcs | 930pcs | 700pcs |
| 40'' HQ Container | 3300pcs | 2300pcs | 1750pcs |