LED High Bay Light-UFO11

Specifications

| Product Series | UFO 11 |
| --- | --- |
| Model No. | PTHE06A0/ PTHE06A5 / PTHE06B0/ PTHE06B4 |
| Version | V3 |
| Date | 2023/3/20 |

Advantages

- Unique flat wheel form design

- Patented ADC12 graded lightweight housing

- Built in driver with 6KV surge protection

- Wide input voltage AC 100-277V

- Optical glass over 90% light transmitting

- LED 2835/3030 chip with low lumen decay

- IP65 driver with short circuit/over-flow/over-temperature/over-voltage protection

- 150lm/W, over 90% energy saving

- Lifespan 50,000 hours, 5 years’ warranty

- Easy installation and low maintenance

Product Size

100W

150W

200-250W

PTHK01B0

Unit: mm

Specifications

| Model No. | PTHE06A0 | PTHE06A5 | PTHE06B0 | PTHE06B4 |
| --- | --- | --- | --- | --- |
| Input Voltage | AC100-277V | AC100-277V | AC100-277V | AC100-277V |
| Power | 100W | 150W | 200W | 240W |
| Dimming | No/1-10V/Microwave Sensor/Lux Sensor | No/1-10V/Microwave Senso/Lux Sensor | No/1-10V/Microwave Sensor/Lux Sensor | No/1-10V/Microwave Sensor/Lux Sensor |
| Spd. | 6kV line-line, 6kV line-earth |  |  |  |
| PF | >0.95 |  |  |  |
| LED Type | 2835/3030（LM80） |  |  |  |
| Beam Angle | 120° |  |  |  |
| Light Efficiency | 150 lm/W |  |  |  |
| Luminous | 15000 lm | 22500 lm | 30000 lm | 36000 lm |
| Color Temp | 3000±200K / 4000±250K / 5700±300K / 6500±500 K |  |  |  |
| CRI | 70/80Ra |  |  |  |
| IP | IP65 |  |  |  |
| Working Temp | -20°C~45°C |  |  |  |
| Storage Temp | -40°C~60°C |  |  |  |
| Life Span | 50000h@25°C |  |  |  |

Installation

Function available

PHOTOCELL AND DAYLIGHT HARVESTING

MOTION SENSOR+DAYLIGHT PRIORITY

Package

| Model No. | PTHE06A0 | PTHE06A5 | PTHE06B0 | PTHE06B4 |
| --- | --- | --- | --- | --- |
| QTY/Carton | 1 pcs | 1 pcs | 1 pcs | 1 pcs |
| Carton Size | 288*288*206mm | 335*335*218mm | 361*361*231 | 361*361*244 |
| N.W | 2.2kg | 2.9kg | 3.5 | 3.8 |
| G.W | 2.7kg | 3.5kg | 4.5 | 4.8 |
| 20''GP Container | 1600pcs | 1120pcs | 920pcs | 860pcs |
| 40''HQ Container | 3980pcs | 2780pcs | 2260pcs | 2140pcs |